README for OLD Grand Average Leg Statistics for Each Day – “old” data

These data were processed (averaged in 1-km segments) in 2005 as described in LeMone et al. (2003) and LeMone et al. (2006).

1-km fluxes were found using the following procedure

- Data detrended for each flight leg to obtain w’,’, and q’

- Fast w and q data shifted to correspond to w’

- 1-km average fluxes found, starting with beginning time of the leg

- 1-km average fluxes interpolated to common points

- Interpolated fluxes averaged.

1-s position data were used, consistent with the estimated location accuracy of 100 m.

Latitude and longitude standard deviations for some days to see

- How close the interpolated averages are to the nominal latitude (for N-S legs – Western Track) or to nominal longitude for E-W (Central Track) or ENE-WSW (Eastern Track) legs.

- How close the pilot was flying to the flight track (longitude for N-S legs), latitude for E-W and ENE-WSW legs.

The averaged 1-km segment data:

- Time – UTC – average times for segments

- Latitude (degrees and fractions of degree) corrected using GPS

- Longitude (degrees and fractions of degree) corrected using GPS

- Ralt = radar altitude (ralt1 or ralt2)

- Palt = pressure Altitude (ztrue)

- H = sensible Heat Flux in Watts per square meter

- LE = latent heat flux in watts per square meter

- Sigma:  standard deviation except that n was used rather than n-1.

References:

LeMone, M.A., R.L. Grossman, F. Chen, K. Ikeda, and D. Yates, 2003:  Choosing the averaging interval for comparison of observed and modeled fluxes along aircraft transects over a heterogeneous surface.  J. Hydrometeor., 4, 179-195.

LeMone, M.A., F. Chen, J.G. Alfieri, M. Tewari, B. Geerts, Q. Miao, R.L. Grossman, and R.L. Coulter, 2005:  Influence of land cover, soil moisture, and terrain on the horizontal distribution of sensible and latent heat fluxes and boundary layer structure in southeast Kansas during IHOP_2002 and CASES-97.  J. Hydrometeorology, in press.

Updated 25 July 2006